### Proposed title

Machine-Learning-Based Prediction of Critical Heat Flux over Engineered Surfaces under Diverse Thermal-Hydraulic Conditions

or

A Generalized Machine-Learning Framework for Critical Heat Flux Prediction over Different Surface Characteristics and Operating Conditions

or

Physics-Guided Machine Learning for Critical Heat Flux Prediction and Regime Identification over Engineered Surfaces

Abstract (250 words, keywords 5-7)

# Introduction

### Nuclear relevance of CHF

Begin from the nuclear engineering perspective.

Discuss: CHF as a limiting heat-transfer phenomenon., Departure from nucleate boiling. Dryout/burnout. Reduction in heat-transfer capability after CHF.

Importance of CHF prediction in:

  - nuclear reactor fuel cooling,

  - pressurized water reactors,

  - boiling water reactors,

  - accident-tolerant cooling concepts,

  - high-heat-flux thermal systems.

Then establish that conventional CHF correlations have limited applicability outside their original database. Generate an image showing last 15 years trends of studies in ML and AI based CHF prediction.

### Influence of surface characteristics

Introduce the importance of engineered surfaces: surface roughness, wettability, contact angle, surface material, surface orientation effects on nucleation-site density, bubble departure, liquid replenishment, microlayer evaporation, vapor escape, dry spot formation, CHF.

### Limitations of existing CHF correlations

This is an important subsection.

Discuss limitations such as:

- empirical nature,

- narrow experimental ranges,

- dependence on specific fluids,

- dependence on particular geometries,

- difficulty incorporating surface characteristics,

- poor extrapolation,

- nonlinear coupling among parameters.

Then introduce ML as a potential solution.

### Existing ML-based CHF prediction

Review:

- ANN,

- Random Forest,

- Gradient Boosting,

- XGBoost,

- Support Vector Regression,

- Gaussian Process Regression,

- Deep Neural Networks.

But identify the gap:

Most ML studies focus primarily on maximizing prediction accuracy, whereas the ability of models to generalize across unseen surface conditions and maintain physical consistency has received comparatively limited attention.

This can become one of your major novelty claims.

### Research gap

I would formulate the gap around four levels:

Gap 1: Existing CHF correlations inadequately account for engineered surface characteristics.

Gap 2: Existing ML models frequently rely on randomly divided datasets, allowing samples from the same surface/operating condition to appear in both training and testing datasets.

Gap 3: Cross-surface generalization remains insufficiently demonstrated.

Gap 4: ML predictions are often treated as black boxes, with limited interpretation of how surface and thermal-hydraulic parameters control CHF.

### Objectives

A strong objective statement explaining the essence of the paper.

# Experimental database and CHF measurements

This section should be extremely detailed because the credibility of the ML model depends on the experimental database.

## Experimental facility, test surfaces, their explanation and tabulations

### Figure 1

Schematic of the experimental CHF facility

# Experimental input variables

I would divide the ML inputs into four groups.

## 6.1 Geometrical parameters

6.2 Operating parameters

## 6.3 Fluid properties

## 6.4 Surface characteristics

# CHF detection criterion

### Criterion 1 — Wall temperature excursion

### Criterion 2 — Sudden wall-temperature increase

### Criterion 3 — Heat-flux reduction

### Criterion 4 — Visual detection

# Experimental uncertainty

Quantify uncertainty in: heat flux, wall temperature, pressure, mass flow rate, surface temperature, CHF determination. of different papers

# Machine-learning methodology

## Data preprocessing

Include:

- missing-value treatment,

- outlier detection,

- normalization/standardization,

- feature correlation analysis,

- dimensionality analysis.

Avoid blindly removing outliers. Explain whether an outlier represents experimental error or a genuine CHF condition.

# ML models

I recommend comparing at least five algorithms.

### Model 1 — ANN

Your primary nonlinear model.

Architecture:

### Model 2 — Random Forest

Useful for:

- nonlinear relationships,

- feature importance,

- robustness.

### Model 3 — XGBoost

Likely to provide strong performance on structured/tabular experimental data.

### Model 4 — “ you say”

### Model 5 — Proposed

# 12. Proposed model hierarchy

# 13. Critical issue: data splitting

This is potentially one of the most important contributions.

Do not rely only on:

80% training + 20% testing randomly.

Because if the same surface has data in both sets, the ML model may simply learn the characteristics of that surface.

Instead use three validation strategies.

### Strategy I — Random split

### Strategy II — Condition-wise split

### Strategy III — Surface-wise split

### Strategy IV — Leave-one-surface-out cross-validation

# 14. Performance metrics comparison of all ml models

### (R2), RMSE, MAE, MAPE, training time

# 15. Results structure Experimental CHF behavior vs predicted for all cases parity plots

# Explainable AI, Figure 4, Global SHAP feature importance

Potential ranking: mass flux, subcooling, contact angle, roughness, pressure, heated diameter, surface structure size, fluid properties.

** all figures should be png with fonts times new roman 18 bold**

**Texts should be times new roman, 12, 1.5 spacing**