# The Physics of Critical Heat Flux — Foundation Document

Project: Generalized CHF prediction across surfaces, geometries and fluids

Purpose: Assemble every physical law, mechanism and closed-form equation that governs

CHF, so that the predictive model can be built *on physics* rather than on curve-fitting.

Status: Research compilation. Section 9 (proposed algorithm) is a discussion draft, not

implemented code.

• • •

## 0. Why this document exists

Everything in this repository so far predicts CHF by *fitting*: look-up-table

interpolation, tree ensembles, ANNs, GPR. The audit in CHF Extrapolation Audit.html

and the unified_chf_pipeline results both converge on the same verdict — these models

interpolate beautifully (R² ≈ 0.96–0.97 on random splits) and collapse the moment they

are asked for a surface, a fluid or a pressure they have not seen (strategy-3 R² = +0.17

to +0.22; LOSO folds near zero or negative).

That failure is not a hyperparameter problem. It is a basis problem. A model whose

features are (P, G, x, D, L) in raw engineering units has no way of knowing that R123

and water are the same physics at different scales. A model whose features are

*dimensionless groups derived from the momentum and energy balances* does.

So this document collects the physics: the conservation laws, the instability criteria,

the trigger mechanisms, the closed-form correlations, and — most importantly for us —

the asymptotic and monotonicity constraints that any admissible CHF model must obey.

Those constraints (Section 8) are the raw material for a physics-constrained algorithm.

A note on rigour. Every equation below is tagged:

| Tag | Meaning |
| --- | --- |
| [V] | Transcribed verbatim from a primary source read during this research; source cited inline. |
| [S] | Structure verified from a secondary source; one or more constants could not be read cleanly and must be checked against the original paper before publication. |
| [X] | Known to exist and be relevant, but not transcribed here because no clean primary source was obtained. Do not cite from memory. |

Nothing in this document was written from recollection. Where a source could not be

obtained, that is stated rather than papered over.

• • •

## 1. The phenomenon

### 1.1 The boiling curve and the definition of CHF

Plotting wall heat flux q″ against wall superheat ΔT_sat = T_w − T_sat produces the

boiling curve, with four regimes separated by three transition points

(Liang & Mudawar 2018, §1.2) [V]:

Single-phase — low superheat, convection only.

Nucleate boiling — bubbles nucleate at the surface; highest heat transfer coefficients.

Transition boiling — parts of the surface nucleate, parts are vapour-blanketed.

Film boiling — continuous vapour blanket; lowest heat transfer coefficients.

Transitions: *onset of boiling* → *CHF* → *minimum heat flux (Leidenfrost)*.

CHF is the point at which bubble nucleation is replaced by localised vapour blankets merging across the surface.

The engineering consequence is asymmetric and is the entire reason CHF matters:

In a temperature-controlled system (fossil boiler tube), crossing CHF moves you

along the curve into transition boiling — recoverable.

In a heat-flux-controlled system (a nuclear fuel rod, a power electronics die),

crossing CHF forces a jump from nucleate boiling straight to film boiling, because

there is no stable intermediate state at fixed q″. The wall temperature excursion

is immediate and can be hundreds of kelvin. This is *burnout* / *boiling crisis*.

NUREG/KM-0011 §2.1 makes the point explicitly: pre-nuclear boiling systems were mostly

temperature-controlled, so CHF was not urgent; water-cooled reactors are heat-flux

controlled and are *power-limited by CHF*. [V]

### 1.2 Two physically distinct crises share one name

This distinction matters enormously for a generalised model, and most ML papers ignore it

(Zhao et al. 2019 note that none of the prior ML publications even attempted to

distinguish them) [V]:

|  | DNB (departure from nucleate boiling) | Dryout |
| --- | --- | --- |
| Regime | Subcooled / low quality | High quality, annular flow |
| Mechanism | Vapour blanket forms *at the wall* while bulk is still liquid | The annular liquid *film* is fully evaporated/entrained |
| Quality at CHF | x < 0 to small positive | x large positive |
| Heat flux level | High | Much lower |
| Excursion | "Fast dryout" — drastic, PWR-like | "Slow dryout" — gradual, BWR-like |
| Reactor | PWR | BWR |

Groeneveld (1986), via NUREG/KM-0011 §2.2, uses exactly this fast/slow dryout

language. [V]

Implication for us: a single smooth function CHF = f(P, G, x, …) is being asked to

represent two different physical mechanisms with different governing balances. Any

generalised model should either (a) carry a regime indicator, or (b) be built as a

mechanism-wise mixture. This is a stronger version of the geometry_family one-hot fix

already made in unified_chf_pipeline/scripts/features.py, which alone moved

helical_coil_r123 from R² = −10.3 to R² = 0.93–0.96.

### 1.3 CHF detection criteria — the target variable is not uniquely defined

The outline's §"CHF detection criterion" needs this. NUREG/KM-0011 §2.2 documents four

historical methods [V]:

Visual — the test section "started to redden visually" (e.g. Hood & Isakoff 1962).

Works for fast DNB; fails for slow BWR-type dryout, which produces only modest

excursions and no discolouration.

Physical burnout — the section actually fails before power can be cut. Common at

high flow / high subcooling where CHF is very large.

Change in test-section resistance — stainless steel (high temperature coefficient

of resistance) used as one leg of a Wheatstone bridge; the excursion unbalances the

bridge and trips the supply. Best for very fast excursions. (Dell et al. 1969;

Matzner et al. 1965; Hewitt et al. 1965.)

Test-section thermocouples — most common. Attached at the downstream end of the

heated length. Unreliable for very slow dryouts, where the better criterion is a

change in the slope ΔT_w/Δq (Groeneveld 1986).

Plus a fifth, "byproduct" case: in film-boiling experiments, CHF quality was either taken

as the quality of first surface-temperature rise, or as the average of the last pre-CHF

and first post-CHF quality (Era et al. 1966; Bennett et al. 1967; Herkenrath et al. 1967). [V]

This is a label-noise source, not a footnote. Rows in our merged dataset carry CHF values defined by different criteria. Merging them without a detection_criterion column means the model is fitting a target whose definition varies by source — and our hardest splits (surface-wise, leave-one-source-out) are *exactly* the splits where the criterion changes between train and test. Our master_chf_dataset.csv has no such column today.

### 1.4 Primary vs secondary parameters

NUREG/KM-0011 §2.3 [V]. CHF is a function of five primary parameters: pressure,

inlet temperature, mass flow, diameter, heated length.

Critically: Lee & Obertelli (1963) and Lee (1965) showed that **heated length and inlet

temperature can be replaced by the thermodynamic quality at the CHF location**, provided

L/D > 50 so upstream history effects are washed out. This is the origin of the

*local conditions hypothesis* (§1.6) and the reason the LUT is indexed on (P, G, x).

Secondary parameters, with NUREG's assessment:

| Parameter | Effect on CHF |
| --- | --- |
| Orientation | Not significant at high flow where stratification is suppressed (Wong et al. 1990); use a flow-regime map (Taitel & Dukler 1975) to bound it. |
| Test-section material | Generally little effect in flow boiling. But at low flow / DNB conditions, high-conductivity materials can suppress hot spots under bubbles and raise CHF. |
| Type of heating (Joule vs nuclear) | No significant effect (Leung et al. 1982). |
| Wall thickness | No discernible effect (Bergles 1963; Bennett et al. 1965), except possibly for very thin walls. Contrast with pool boiling, §4.5 — there it matters a lot. |
| Surface roughness | Generally small in flow boiling, *"because the vapor generation rate at the surface usually determines the CHF occurrence."* But: when roughness exceeds the liquid film thickness in annular-film dryout, premature film breakdown reduces CHF. |
| Inlet/outlet throttling | Outlet restriction → flow/pressure instability → significant CHF reduction (Lowdermilk 1958; Mayinger et al. 1966). A "soft" (unthrottled) inlet permits oscillations; a "hard" inlet suppresses them (Kirillov 1997). |
| Dissolved gas | Up to 30% CHF reduction at 4,000 Ncm³/kg (Kirillov 1997). |

Note the tension with the outline's premise. NUREG says roughness effects in *flow* boiling are usually small; the pool-boiling literature (§4.2) says roughness and wettability matter enormously. Both are right — they are different regimes. The defensible framing for the paper is: *surface characteristics dominate in pool boiling / low-flow DNB and in annular dryout when roughness is film-scale, and are second-order in high-flow subcooled DNB.* A generalised model must reproduce that regime-dependence, not assert a single global surface sensitivity.

### 1.5 Energy balance and thermodynamic quality

The one exact relation in the whole subject. For a uniformly heated tube, an energy

balance between inlet and the CHF location (Hall & Mudawar 2000b, Eq. 8) [V]:

h_o = h_i + 4 · (L/D) · (q″/G)

In non-dimensional form (their Eq. 9–10) [V]:

x_o = x_i* + 4 · Bo · (L/D)

where the boiling number is

Bo = q″ / (G · h_fg)

and x_i* is the *pseudo-inlet quality* (h_i − h_f,o)/h_fg,o, i.e. inlet enthalpy

referenced to *outlet*-pressure saturation properties — which is what lets the

correlation avoid needing the inlet pressure at all.

Thermodynamic equilibrium quality:

x = (h − h_f) / h_fg

with saturated properties evaluated at the pressure of the CHF point (usually outlet).

x < 0 denotes subcooled conditions and is a *pseudo*-quality — there is no vapour, it is

an enthalpy deficit expressed in quality units.

Why this matters to us: this equation is an exact constraint linking

x_o, x_i, L/D, Bo. It means an "inlet conditions" correlation and an "outlet (local)

conditions" correlation are transformations of one another, *not* independent modelling

choices. Any model that takes x, L/D and G as free independent features is

implicitly allowed to violate energy conservation. **This is a hard constraint we can

impose for free.**

### 1.6 The local conditions hypothesis

For a water-cooled tube of fixed diameter, CHF is a unique function of the *local* pressure, mass flux and thermodynamic quality at the CHF location.

(NUREG/KM-0011 §3.3 [V].) This is the assumption underpinning the entire look-up

table approach. It is an *assumption*, and it is what fails when L/D < 50, under

non-uniform axial heating, and under flow instability. Hall & Mudawar note the

consequence precisely: the outlet-conditions correlation *cannot* show an L/D effect,

because it hypothesises CHF depends only on local conditions — whereas the inlet-conditions

form does exhibit L/D. [V]

• • •

## 2. Dimensional analysis — the backbone of generalisation

This section is the most important one for our modelling goal.

### 2.1 Kutateladze's derivation (1948) — worked through

The archetype for how *all* of this works. From Liang & Mudawar 2018 §1.4 [V]:

Step 1. Vigorous boiling releases vapour perpendicular to the surface at velocity

u_g = q″ / (ρ_g · h_fg)                                              (1)

Step 2. At CHF, the vapour's kinetic energy just balances the gravitational force on

the suspended liquid:

ρ_g · u_g²  ~  g · (ρ_f − ρ_g) · δ*                                  (2)

Step 3. δ* is the linear scale of capillary disturbances — the capillary length:

δ* = [ σ / (g(ρ_f − ρ_g)) ]^(1/2)                                    (3)

Step 4. Substituting (1) and (3) into (2) yields the Kutateladze–Zuber form:

q″_CHF
──────────────────────────────────  =  K                             (4)
ρ_g h_fg [ σ g (ρ_f − ρ_g) / ρ_g² ]^(1/4)

Kutateladze recommended K = 0.16 for a large horizontal flat surface.

The key structural insight, stated by Liang & Mudawar themselves:

"because this equation was derived from dimensional analysis premises, it is expected that different theoretical CHF models for saturated pool boiling may be arranged into the form of Eq. (4), independent of the CHF mechanism proposed." [V]

So the entire pool-boiling literature reduces to *one functional form and a dimensionless

number K*, where K carries the physics of the mechanism, the surface, the orientation and

the fluid. **That is the template for our model: predict a dimensionless CHF, not a

dimensional one.**

### 2.2 The dimensionless group inventory

| Group | Definition | Physical meaning | Source |
| --- | --- | --- | --- |
| Dimensionless CHF K | q″/(ρ_g h_fg [σg(ρ_f−ρ_g)/ρ_g²]^{1/4}) | CHF vs. hydrodynamic limit | Kutateladze [V] |
| Boiling number Bo | q″/(G h_fg) | Wall evaporation vs. total flow enthalpy capacity | Hall & Mudawar [V] |
| Weber number We_D | G²D/(ρ_f σ) | Flow inertia vs. surface tension | Hall & Mudawar [V] |
| Katto number | σρ_f/(G²L) | Inverse Weber on heated length | Katto & Ohno [V] |
| Density ratio | ρ_f/ρ_g or ρ_g/ρ_f | Proxy for reduced pressure; sets regime boundaries | multiple [V] |
| Reduced pressure | P/P_crit | Property-collapse coordinate | multiple [V] |
| Capillary length | [σ/(g(ρ_f−ρ_g))]^{1/2} | Bubble/instability length scale | Kutateladze [V] |
| Bond number | Bo_d = (ρ_l−ρ_g) g d²/σ | Buoyancy vs. surface tension | Merilo, via Hardik & Prabhu [V] |
| Jakob number | Ja = ρ_f c_p ΔT_sub/(ρ_g h_fg) | Sensible vs. latent capacity (subcooling) | present in master_chf_dataset.csv col ja |
| Kandlikar K₁ | (q″/(G h_fg))² · (ρ_f/ρ_g) | Evaporation momentum vs. inertia | Kandlikar [S] |
| Kandlikar K₂ | (q″/h_fg)² · D/(ρ_g σ) | Evaporation momentum vs. surface tension | Kandlikar [S] |
| Thermal activity S | H·√((ρ c_p k)_w) | Wall thermal inertia per unit area | Bar-Cohen & McNeil [V] |
| L/D | — | Heated-length / history effect | multiple [V] |
| D/D_ref | D/8mm | Diameter normalisation | Groeneveld LUT [V] |

Kandlikar's interrelations [S]: K₂/K₁ = We, K₃/K₁ = Re, K₂/K₃ = Ca, where K₃

is evaporation momentum over viscous force. The general dimensionless statement of a tube

CHF correlation, from NUREG/KM-0011 §3.1 [V]:

CHF                ⎛ ρ_f        D^0.5              D    ⎞
─────────  =  f      ⎜ ───  ,  ───────────  ,  x  ,  ──── ⎟
 h_fg G              ⎝ ρ_g     (σρ_f)^0.5           D_ref ⎠

i.e. Bo = f(density ratio, Weber-like group, quality, geometry ratio).

This is the single most actionable equation in this document. It says the *correct* feature space for a generalised CHF model is four dimensionless numbers, not five dimensional ones. It is fluid-agnostic by construction: water and R123 map to the same point if their dimensionless groups match. Our current pipeline already takes a half-step toward this by training on log(CHF / physics_baseline) — which is log(Bo/Bo_ref) in disguise, and which is exactly what moved strategy-3 from R² = −16.4 to +0.22. Section 9 argues for taking the full step.

• • •

## 3. Pool boiling CHF — the five trigger mechanisms

Liang & Mudawar 2018 §1.3 identify five competing mechanisms [V]. There is *still* no

consensus on which is correct — that lack of consensus is itself a research finding worth

stating in the paper.

### 3.1 Bubble interference (Rohsenow & Griffith 1955)

CHF when neighbouring isolated bubbles coalesce radially [V]:

q″_CHF = 0.012 · ρ_g · h_fg · [ (ρ_f − ρ_g)/ρ_g ]^0.6              (5)

Chang & Snyder's variant, which introduces contact angle α — historically the first

appearance of a surface property in a CHF model [V]:

q″_CHF = ½ (π/6)^(5/6) (0.0119 α)^(1/2) · ρ_g h_fg [2σg(ρ_f−ρ_g)/ρ_g²]^(1/4)   (6)

Both are undermined by later high-speed photography showing coalescence occurs *well

before* CHF.

### 3.2 Zuber hydrodynamic instability (1959) — full derivation

The most influential CHF model ever published. Full chain from Liang & Mudawar §3.1 [V]:

Geometry. Vapour jets leave the surface at u_g; liquid returns between them at u_f.

The surface is modelled as repeating square cells, one jet each. Jet diameter is set by

Rayleigh–Taylor instability: D_j = λ_T/2, where λ_T lies between:

critical wavelength:        λ_c = 2π √( σ / (g(ρ_f − ρ_g)) )              (7a)
most dangerous wavelength:  λ_d = √3 · λ_c = 2π√3 √( σ/(g(ρ_f−ρ_g)) )     (7b)

Area ratio. For a square cell of side λ_T containing one jet of diameter λ_T/2:

A_g/A_w = π(λ_T/2)²/4 / λ_T² = π/16                                        (8)

Continuity.

u_f = (ρ_g/ρ_f) · [ (A_g/A_w)/(1 − A_g/A_w) ] · u_g
    = (ρ_g/ρ_f) · [ π/(16 − π) ] · u_g                                     (9)

Energy. All wall heat goes to vaporisation:

q″_CHF = (A_g/A_w) ρ_g h_fg u_g       ⟹      u_g = q″_CHF / ((π/16) ρ_g h_fg)   (10, 11)

Trigger — Kelvin–Helmholtz instability. The velocity difference across the jet

interface destabilises it; growth merges adjacent jets into a vapour mushroom that blocks

liquid resupply:

u_g − (−u_f) = [ (ρ_f + ρ_g)/(ρ_f ρ_g) ]^(1/2) · ( 2πσ/λ_H )^(1/2)         (12)

with Zuber's closure λ_H = π D_j (13).

Result. Combining (7a)–(13) [V]:

q″_CHF                       ⎡ π    3    1   ⎤   ⎡      (1 + ρ_g/ρ_f)^(1/2)  ⎤
──────────────────────────────  =    ⎢ ── ───── ──── ⎥ · ⎢ ───────────────────────── ⎥
ρ_g h_fg [σg(ρ_f−ρ_g)/ρ_g²]^(1/4)    ⎣ 24 √(2π)  3^¼ ⎦   ⎣    1 + ρ_g/ρ_f · π/(16−π) ⎦

The bracket range (from λ_c < λ_T < λ_d) gives K between 0.119 and 0.157; Zuber

recommended the intermediate K = 0.131 [V]:

q″_CHF = 0.131 · ρ_g h_fg [ σ g (ρ_f − ρ_g)/ρ_g² ]^(1/4)                  (15)

This is the zuber_pool_boiling_chf() already implemented in scripts/chf_physics.py

and used as the pool-boiling half of compute_physics_baseline_kw_m2 in the unified

pipeline.

Note what Zuber's model does *not* contain: any surface property. No contact angle,

no roughness, no material, no wettability. That absence is the launching point for the

entire engineered-surface literature (§4) and for the outline's Gap 1.

### 3.3 Lienhard & Dhir finite-size correction

Setting λ_H = λ_d instead of Zuber's λ_H = πD_j [V]:

q″_CHF / q″_CHF,Zuber = 1.14        ⟹     K = 0.149                       (16)

Validity: the infinite-flat-plate assumption holds for surface widths > 3λ_d

(Lienhard et al.). Zhang et al. (2D simulation): the surface can be treated as infinite

when heater width / capillary length > 12. Gogonin: 2 capillary lengths suffice. [V]

Directly relevant to our pin-fin and flat-heater pool-boiling sources. A finite heater is not a small perturbation on Zuber — it is a different asymptote. If heater_width_mm / capillary_length < 3λ_d, the Zuber baseline is the wrong normaliser and our log(CHF/baseline) target inherits that error.

### 3.4 Haramura & Katto macrolayer dryout (1983)

A large hovering bubble sits above a thin liquid macrolayer pierced by vapour stems.

CHF is triggered when the macrolayer dries out just before bubble departure [V]:

q″_CHF = ρ_f h_fg δ (1 − A_g/A_w) f                                       (25)

δ = macrolayer thickness, f = bubble departure frequency. With δ = λ_H/4 [V]:

q″_CHF = 0.721 (A_g/A_w)^(5/8) [ (1 − A_g/A_w)^(5/16) ((ρ_f/ρ_g)+1) / ((11/16)(ρ_f/ρ_g)+1)^(3/5) ]^(5/16)
         × ρ_g h_fg [ σ(ρ_f−ρ_g)g/ρ_g² ]^(1/4)                            (26a)

with

A_g/A_w = 0.0584 (ρ_g/ρ_f)^(1/5)                                          (27)

Macrolayer thickness — four competing closures [V]:

| Author | δ |
| --- | --- |
| Haramura & Katto | λ_H/4 = 0.00536 σ ρ_g (1+ρ_g/ρ_f)(ρ_g/ρ_f)^{2/5} (h_fg/q″)² |
| Rajvanshi et al. | λ_H/2 = 0.0107 σ ρ_g (1+ρ_g/ρ_f)(ρ_g/ρ_f)^{2/5} (h_fg/q″)² |
| Kumada & Sakashita | 0.786 [ν_g⁸σ¹¹/(ρ_f⁶g⁵(ρ_f−ρ_g)⁵)]^{1/24} (ρ_g h_fg/q″)^{5/6} |
| Chappidi et al. | [ (ρ_g/ρ_f)(A_g/A_w)(1−A_g/A_w)^{−1} √((ρ_f+ρ_g)/(ρ_fρ_g)) √σ τ ]^{2/3}, τ = bubble hovering time |

Also δ = 1.1 R (R = mean vapour stem radius), Chappidi et al. [V]

Status honestly reported: Xiao & Yu report the macrolayer never dries out; Jung et al.

found *no* evidence of regularly spaced vapour jets or a trapped liquid layer using

high-speed IR. Those observations undermine both the macrolayer model *and* Zuber. [V]

### 3.5 Hot / dry spot models

Yagov (1988, 2014) — CHF as irreversible growth of dry spots. Uniquely among these

models, it carries liquid viscosity dependence.

Low reduced pressure, P/P_c < 0.001 [V]:

0.5 · h_fg^(8/55) σ^(9/11) ρ_g^(13/110) k_f^(7/110) g^(21/55) f(Pr_f)
q″_CHF,l  =  ─────────────────────────────────────────────────────────────────────   (33)
                        ν_f^(1/2) c_p,f^(3/10) R_i^(79/110) T_sat^(21/22)

Pr_f^(9/8)
f(Pr_f) = [ ──────────────────────────────── ]^(4/11)                     (34)
            1 + 2Pr_f^(1/4) + 0.6 Pr_f^(19/24)

High reduced pressure, P/P_c > 0.03 [V]:

q″_CHF,h = 0.06 h_fg ρ_g^(3/5) σ^(2/5) [ g(ρ_f − ρ_g)/μ_f ]^(1/5)         (35)

Blend for 0.001 ≤ P/P_c ≤ 0.03 [V]:

q″_CHF = ( q″_CHF,h³ + q″_CHF,l³ )^(1/3)                                  (36)

This blending function is a template worth stealing. It is a smooth, differentiable, physically-motivated way to interpolate between two asymptotic regimes — far better behaved than the hard if switches in Biasi and Katto–Ohno, and directly applicable to our mechanism-mixture idea in §9.

Theofanous & Dinh — CHF governed by microlayer instability [V]:

q″_CHF = λ^(−1/2) · ρ_g h_fg [ σ g (ρ_f − ρ_g)/ρ_g² ]^(1/4)               (37)

λ decreases with increasing wettability. Kim et al. derived it from Rayleigh's static

meniscus volume formula, for hydrophilic surfaces (α < 90°) [V]:

λ = [ (1 − sin α)/2 − (π/2 − α)/(2 cos α) ]^(−1/2)                        (38)

### 3.6 Galloway & Mudawar interfacial lift-off

CHF occurs when vapour momentum normal to the wall exceeds the restraining pressure force

from interfacial curvature; wetting fronts in the wave troughs are the last cooling

mechanism [V]:

ρ_g [ q″_l / (ρ_g h_fg (1 + c_p,f ΔT_sub/h_fg)) ]²  =  P_f − P_g          (39)

P_f − P_g = 2√2 π σ δ / λ_c²                                              (40)

With wetting-front span = λ_c/4 (so q″_CHF = q″_l/4) [V]:

q″_CHF = ¼ ρ_g h_fg (1 + c_p,f ΔT_sub/h_fg) · [ 2√2 π σ δ / (ρ_g λ_c²) ]^(1/2)   (41)

This model spans pool *and* flow boiling — Mudawar's group found near-vertical pool

boiling interfacial behaviour closely resembles flow boiling CHF. Note the explicit

subcooling factor `(1 + c_p,f ΔT_sub/h_fg)` — this is the canonical way subcooling

enters CHF models, and it is a Jakob-number term.

• • •

## 4. Surface characteristics — the physics behind the outline's core claim

This is the section the paper outline is built on. The honest summary: **the literature

is genuinely contested, and that contest is itself the research opportunity.**

### 4.1 Contact angle and wettability

The disagreement, stated plainly (Liang & Mudawar §3.2.2) [V]:

*Against:* Kutateladze–Zuber is independent of surface conditions and Bewilogua et al.

validated that independence. Stock: contact angle has only weak influence. O'Hanley et

al.: wettability alone has little influence on smooth surfaces.

*For:* Gaertner; Costello & Frea; Maracy & Winterton; Hahne & Diesselhorst all found CHF

decreases appreciably with decreasing wettability. Liaw & Dhir: at a contact angle of

107°, CHF is only half the hydrodynamic-theory prediction.

*Caution:* Liao et al. warn a robust CHF–contact-angle relation is hard to establish

because fluid and surface thermal properties vary synchronously with temperature, which

itself moves the contact angle.

Liang & Mudawar's conclusion [V]:

"contact angle is important to modeling pool boiling CHF even for smooth surfaces, let alone the complicated contact angle variations resulting from external influences. This fact points to a need for more systematic studies of pool boiling CHF from smooth surfaces."

Models incorporating contact angle α:

| Author | K | Notes |
| --- | --- | --- |
| Kirichenko & Chernyakov | K = 0.171(1 + 0.324×10⁻³α²)^{1/4} / (0.018α)^{1/2} | [V] |
| Ramilison et al. | K = 0.0044 (π − α)³ R_a^{0.125} | [V] contact angle *and* roughness |
| Kim et al. | see §4.2 Eq. (21) | [V] angle + roughness + peak spacing |
| Kandlikar | see §4.3 | [V] angle + orientation |
| Theofanous & Dinh / Kim | Eq. (38) above | [V] |

Note α in these expressions is in degrees for Kirichenko & Chernyakov and Ramilison

(the (π − α)³ form implies radians for Ramilison — **this unit ambiguity must be

resolved against the primary sources before use**).

### 4.2 Surface roughness

Same pattern of disagreement [V]:

*No effect:* O'Hanley et al., Berenson, Lyon, Ramilison & Lienhard, Nishio & Chandratilleke.

*Effect:* Bailey et al. (CHF above Eq. 15 prediction, attributed to roughness);

Guan et al. (up to 15% enhancement at R_a = 5 µm vs. smooth, for pentane/hexane/FC-72).

*Strong effect:* Kim et al., copper, water, moderate wettability α = 60–70°: CHF rose

from 77.5 W/cm² at `R_a` = 0.041 µm to 162.5 W/cm² at `R_a` = 2.36 µm — a 2.1×

increase, ascribed to capillary wicking on the rougher surface.

*Saturating effect:* Kim et al. on superhydrophilic aluminium (α = 7–16.3°): CHF rose

from 165 to 215 W/cm² between R_a = 0.11 and 0.35 µm, then became **negligibly

dependent** on roughness over 0.35 < R_a ≤ 2.93 µm.

Kim et al. combined roughness + wettability correlation — the single most useful

closed form for the outline's purposes [V]:

⎧ (1 + cos α)  ⎡ 2     π                       cos α    R_a ⎤ ⎫^(1/2)
K = 0.811⎨ ─────────── ⎢ ─  +  ─ (1 + cos α) + 351.2 ─────────  ─── ⎥ ⎬
        ⎩      16      ⎣ π     4                     1 + cos α   S_m ⎦ ⎭     (21)

where S_m is the mean spacing between roughness peaks. Note this needs *two*

roughness descriptors (R_a and S_m), not one — amplitude alone is insufficient.

Data gap in our repository. master_chf_dataset.csv has roughness_factor at 0.6% coverage and no S_m, no contact angle column at all. The outline's Gap 1 and the SHAP ranking it proposes (contact angle, roughness among the top features) are not currently supportable from our merged data. This is the single largest gap between the outline's ambition and the dataset's reality, and it should be resolved before the results section is written.

### 4.3 Orientation

Consistent finding across Lyon, Priarone, El-Genk & Bostanci, Beduz et al., Brusstar et

al. [V]: CHF decreases *slightly* from θ = 0° to 90°, then rapidly toward 180° as

buoyancy traps vapour against the surface.

Howard & Mudawar's photographic study identifies three mechanism regions [V]:

θ = 0–60° (upward-facing): buoyancy removes vapour vertically.

θ = 60–165° (near-vertical): a wavy liquid–vapour interface propagates along the surface.

θ > 165° (downward-facing): vapour stratification, greatly reduced CHF.

Their conclusion is important for us: *"it is impossible to account for orientation

effects using a single CHF model,"* and three separate models should be developed. [V]

Yang et al. also found a transition angle beyond which CHF drops sharply, which

*increases* with heater size while CHF *decreases*.

Orientation correlations for `K` (Liang & Mudawar Table 2, θ in degrees) [V]:

| Author | K | Range |
| --- | --- | --- |
| Kandlikar | K = [(1+cos α)/16] · [ 2/π + (π/4)(1 + cos α) cos θ ]^{1/2} | θ = 0–90° |
| Liao et al. | K = 0.131[−0.73 + 1.73/(1+10^{−0.021(185.4−θ)})]·[1 + ((55−α)/100)(0.56 − 0.0013θ)] | θ = 0–180° |
| Priarone | K_FC-72 = 0.165 f(θ), K_HFE-7100 = 0.21 f(θ), f(θ) = 1 − 0.001117θ + 7.79401×10⁻⁶θ² − 1.37678×10⁻⁷θ³ | θ = 0–175° |
| El-Genk & Bostanci | K = [(0.229 − 4.27×10⁻⁴θ)^{−6} + (0.577 − 2.98×10⁻³θ)^{−6}]^{−1/6} | θ = 0–180° |
| Vishnev | K = 0.0125(190 − θ)^{1/2} | θ = 0–180° |
| Arik & Bar-Cohen | K = 0.131(1 − 0.001117θ + 7.79401×10⁻⁶θ² − 1.37678×10⁻⁷θ³) | θ = 0–180° |
| Chang & You | q″/q″_max = 1 − 0.0012 θ tan(0.414θ) − 0.122 sin(0.318θ) | θ = 0–180° |
| El-Genk & Guo | K_water = 0.034 + 0.0037(180−θ)^{0.656}; K_N2 = 0.033 + 0.0096(180−θ)^{0.479}; K_He = 0.002+0.0051(180−θ)^{0.633} | θ = 90–180° |
| Brusstar & Merte | `K = (π/24)\ | sin θ\ |

The Kandlikar model is the one to build on — it is the only entry that is

simultaneously theoretical (a force balance on a bubble: evaporation momentum vs. surface

tension vs. hydrostatic) and carries *both* the receding contact angle α and the

orientation θ. Our dataset has orientation and angle_deg columns (0.2% coverage).

### 4.4 Wickability — the modern paradigm

Rahman, Ölçeroğlu & McCarthy engineered ~40 surfaces of varying wickability and showed

CHF increases linearly with surface wickability (wicked volume flux), reaching

260 W/cm² on the highest-wickability hierarchical structure. The controlling

mechanism is capillary-driven liquid supply to the surface, not contact angle per se.

Surfaces studied included biotemplated nanostructures, square micropillar arrays of

varying diameter/height/pitch, and hierarchical structures. [S] — obtained via search

summary; the primary papers (Rahman et al., *Langmuir* 2014; *Sci. Rep.* 2017) were

paywalled/CAPTCHA-blocked and **the linear coefficient was not obtained. Must be read

before citing a number.**

This matters because it *reconciles* §4.1 and §4.2: contact angle and roughness are both

imperfect proxies for the thing that actually matters — the surface's ability to pull

liquid back to the dry spot. Kim et al.'s roughness result was already ascribed to

wicking. Our dataset's porosity, mbl_lateral_um, mbl_total_um, fin_spacing_um

columns are wickability-relevant geometry, at 0.6% coverage.

### 4.5 Wall thermal properties — the "thermal activity" parameter

Unlike flow boiling (§1.4), in pool boiling the wall itself matters. Bar-Cohen & McNeil

define [V]:

S = H · √( (ρ c_p k)_w )

H = wall thickness. Then:

q″_CHF/q″_CHF,asy = S/(S + 0.8)     (Bar-Cohen & McNeil)      (17)
q″_CHF/q″_CHF,asy = S/(S + 0.1)     (Watwe & Bar-Cohen)       (18)
q″_CHF/q″_CHF,asy = 1 − exp[ −(S/2.44)^{0.8498} − (S/2.44)^{0.0581} ]   (Golobič & Bergles, sat. FC-72)  (20)

Eq. (17) gives 90% of asymptotic CHF at S = 8, 99% at S = 85; Eq. (18) at S = 1 and 10.

Thresholds: stainless steel needs ≥ 0.8 mm to be free of heat-capacity effects

(Tachibana et al.); copper in liquid helium saturates at 0.35 mm (Grigoriev et al.). [V]

Watwe & Bar-Cohen composite, combining hydrodynamics, conduction, heater size and

subcooling — a good example of a fully-assembled K [V]:

π   ⎛   S    ⎞ ⎧          ⎡        ⎛ g(ρ_f − ρ_g) ⎞^(1/2) ⎤ ⎫ ⎧      ⎛ρ_f⎞^(3/4) c_p,f      ⎫
K =  ── · ⎜ ────── ⎟ ⎨ 1 + ⎢0.3014 − 0.01507 L ⎜ ──────────── ⎟     ⎥ ⎬ ⎨1 + 0.03⎜───⎟      ──── ΔT_sub ⎬
     24   ⎝ S+0.1  ⎠ ⎩          ⎣        ⎝      σ       ⎠      ⎦ ⎭ ⎩      ⎝ρ_g⎠      h_fg      ⎭   (19)

Our dataset has surface_material at 0.6% coverage — enough to *identify* material but

not to compute S (no wall thickness column).

### 4.6 Pressure and viscosity effects on K

[V] Deev et al.: helium data fit Eq. (4) with K = 0.2 for P/P_c ≤ 0.75; above

0.75 the Kutateladze–Zuber form breaks down entirely. Labuntsov et al.: water and ethanol

show (P/P_c)^{0.15} dependence. Dhir and Wang et al. independently place the CHF

maximum at `P/P_c ≈ 0.35`.

Bewilogua (cryogens):  q″/q″_max = 0.421 + 3.58(P/P_c) − 6.19(P/P_c)² + 2.21(P/P_c)³   (22)
Wang et al.:           K = 0.18 − 0.14 (P/P_c)^{5.68}                                   (23)
Soziev & Khrizolitova (very low P):  K = 0.16{1 + [σg(ρ_f−ρ_g)]^{1/2}/P}^{1/2}         (24)
Borishanskii (viscosity): K = {0.13 + 4[ρ_f σ^{3/2}/(μ_f² g(ρ_f−ρ_g))^{1/2}]}^{−2/5}   (Table 1)

The `P/P_c ≈ 0.35` maximum is a hard, checkable constraint. Any model we build must reproduce a non-monotonic pressure trend with a peak near P/P_c = 0.35 (≈ 7.7 MPa for water). This is precisely the trend the existing repo notes the CoolProp-based Zuber implementation reproduces at 6–7 MPa without tuning. It is a free validation test, and a model that gets it wrong is wrong regardless of its R².

Sakashita's observation is subtle and worth noting: Kutateladze–Zuber captures the

pressure trend for ethanol and R-141b by tuning K, but underestimates it for water —

because water's wettability *improves with increasing pressure* while ethanol's and

R-141b's does not. Pressure and surface effects are coupled, not separable. [V]

• • •

## 5. Flow boiling CHF — correlations

### 5.1 Why there are 500+ of them

NUREG/KM-0011 §3.1 [V]: Clerici (1966) listed 50+ CHF correlations for water-cooled

tubes; the count now exceeds 500. The reason is stated precisely:

"The choice of correlation parameters (P, G, X_CHF, D) is essentially correct, but the functional relationship between these parameters varies with flow conditions — hence, the large proliferation of correlations."

And bluntly:

"none of the early flow boiling CHF correlations for tubes carry much credibility today, partially because the pool boiling CHF equations had a physical basis, whereas the tube CHF correlations were virtually all empirical."

NUREG singles out Katto (1992) and Lee & Mudawar (1988) for special mention

because they (1) have a phenomenological basis, (2) rest on a large database, and (3) have

wider validity. That is a direct endorsement of the physics-first strategy.

This paragraph is the literature justification for the whole project. It is the

strongest available citation for the outline's "Limitations of existing CHF correlations".

### 5.2 The look-up table method

The generalisation response to the proliferation problem. Built on the local conditions

hypothesis (§1.6). The 2006 LUT covers **P = 0.1–21 MPa, G = 0–8,000 kg m⁻² s⁻¹,

x = −0.50 to 1.00, normalised to a vertical 8 mm water-cooled tube**, with linear

interpolation between entries. [V]

NUREG's stated advantages [V]: greater accuracy, wider applicability, correct

asymptotic trends, less computing time, easily updated.

Lineage: Doroshchuk et al. (1975, 5,000 points) → Groeneveld AECL-UO 1986 (15,442 points)

→ Kirillov et al. → 1995 LUT (combined ~24,000 de-duplicated points, with Huang & Cheng

1994 smoothing to remove sharp variations at data/extrapolation boundaries) → 2006 LUT

(+27 further datasets). [V]

Important caveat for our project, already correctly identified in the pipeline README: the LUT is *smoothed and partly correlation-filled*, not raw measurement. The unified_chf_pipeline deliberately excludes it from the merged training data for this reason. It remains a legitimate baseline and a legitimate normaliser, but not a source of independent data points.

### 5.3 LUT correction factors K1–K8

The mechanism by which the 8 mm-tube table is extended to real geometries. From the VVER

methodology paper (arXiv:2203.15048), citing IAEA-TECDOC-1203 and Groeneveld [V]:

CHF_bundle = K1 · K2 · K3 · K4 · K5 · K6 · K7 · K8 · CHF_table                (8)

The factors assume independence, which the source explicitly calls "a first-order

approximation."

|  | Description | Formula |
| --- | --- | --- |
| K1 | Tube diameter | K1 = (8/D_h)^n for 2 ≤ D_h ≤ 25 mm; = 0.57 for D_h > 25 mm (9) |
| K2 | Bundle geometry | `K2 = min[1, (0.5 + 2(s−d)/d)·exp(−0.5\ |
| K3 | Mid-plane spacer (CANDU) | K3 = 1 + A exp(−0.1 L_s/D_h), A = 1.5 ξ^{0.5}(0.001G)^{0.2} (12,13) |
| K4 | Heated length | K4 = 1 for L/D_h < 5; = exp[e^{2α} L/D_h] for L/D_h ≥ 5, with α = xρ_f/(xρ_f + (1−x)ρ_g) (14,15) |
| K5 | Axial flux distribution | K5 = 1 for x ≤ 0; = q_local/q_BLA for x > 0, q_BLA = (1/L_B)∫q(z)dz over the boiling length (16,17) |
| K6 | Radial flux distribution | K6 = 1 for x ≤ 0; = q(z)_max/q(z)_av for x > 0 (18) |
| K7 | Flow orientation | K7 = 1 − exp[−(A/3)^{0.5}], A = ((1−x)/(1−α))² fG²/(g D_h ρ_f(ρ_f−ρ_g)α^{0.5}) (19,20) |
| K8 | Vertical low flow | for −400 < G < 0: `K8·CHF_table = 2CHF_p − CHF(\ |

Note K3 = 1 and K5 = K6 = 1 for subcooled conditions; K7 = 1 for horizontal flow;

K8 = 1 for upward flow. IAEA-TECDOC-1203 recommends using either K3 or K4, not both.

The source also warns **K2's formula is based on saturated qualities and has not been

tested for negative subcooled qualities. [V]**

The diameter exponent `n` is contested — the single most-studied correction:

Groeneveld originally: n = 0.5 (used to normalise the LUT itself).

Others: n = 1/3.

Biasi's correlation: 0.4 for D < 10 mm, 0.6 otherwise.

Wong correlation (lowest RMS per Tanase et al.): n = 0.58[1 − 0.25exp(−2x)](1 − 15D_h^{−6}G) [V]

Tanase et al. (2009) tabulate n by (P, G, x) region — including negative values

(n = −0.2 to −0.3) at low mass flux, meaning the diameter trend *reverses*. [V]

Tanase et al. Table 3 — `n` by regime [V]:

| P (kPa) | G (kg m⁻²s⁻¹) | x: −0.5..−0.25 | −0.25..0 | 0..0.5 | 0.5..1 |
| --- | --- | --- | --- | --- | --- |
| 100–14000 | 0–250 | −0.2 | −0.2 | −0.2 | −0.3 |
|  | 250–3000 | 0.4 | 0.4 | 0.5 | 0.6 |
|  | 3000–8000 | 0.3 | 0.3 | 0.4 | 0.4 |
| 14000–21000 | 0–250 | −0.2 | −0.2 | −0.2 | −0.3 |
|  | 250–3000 | 0.4 | 0.2 | 0.4 | 0.4 |
|  | 3000–8000 | 0.3 | 0.2 | 0.2 | 0.2 |

Boltenko's physical explanation [V]: at low quality (x < 0.2) CHF increases with

tube ID; at high quality the trend reverses. Tanase's sensitivity study found constant

n = 0.4–0.5 predicts the whole LUT range satisfactorily (RMS 7.1–7.3% for HBM).

Directly usable. This table is a ready-made, physics-derived, regime-dependent feature transform. Rather than letting a model learn the diameter effect from scratch — which it demonstrably cannot do across held-out geometries — we can normalise every row to 8 mm using K1 with a regime-appropriate n, and let the model learn only the residual. This is the same "divide by a physics baseline first" trick that already rescued strategy 3, applied to geometry instead of fluid.

### 5.4 Biasi et al. (1967)

As implemented in scripts/chf_physics.py (citing Todreas & Kazimi, *Nuclear Systems I*).

Units: D in cm, G in g cm⁻² s⁻¹, P in atm, output kW/m². [V] (as implemented)

q_low  = (1.883e4 / (D^α G^(1/6))) · (A/G^(1/6) − x)
q_high = (3.78e4 · B / (D^α G^0.6)) · (1 − x)
CHF    = max(q_low, q_high)

α = 0.6 for D < 1 cm, else 0.4
A = 0.7249 + 0.099 P exp(−0.032 P)
B = −1.159 + 0.149 P exp(−0.019 P) + 8.99P/(10 + P²)

Structural note for modelling: the G^(1/6) terms in the denominator **diverge as

G → 0**, which is why the repo's hybrid_reference_chf dispatches to Zuber at G = 0.

That is a correctness requirement, not a nicety — and it is a concrete example of why a

naive "just use a correlation as a prior" approach breaks at regime boundaries.

### 5.5 Katto & Ohno (1984) generalised correlation

The most physically-grounded widely-used flow-boiling correlation; derived by *vectorial

dimensional analysis* postulating hydrodynamic control, yielding four regimes (L, H, N,

HP). Recommended by the NASA cryogenic study as the best performer for LH₂ flow boiling

(91 points, within a factor of 2 more than 90% of the time). [V] (transcribed from

NASA NTRS 20230009827)

Master form, with inlet subcooling:

q″_CHF = q″_co [ 1 + K(h_f,sat − h_f)/h_fg ]                               (8)

Sub-correlations (We⁻¹ ≡ σρ_f/(G²L)):

q″_co,2  = C_Kc (σρ_f/(G²L))^{0.043} (D_H/L) G h_fg                        (9)   [S: C_Kc]

q″_co,3  = 0.1 (ρ_g/ρ_f)^{0.133} (σρ_f/(G²L))^{0.333} · G h_fg / (1 + 0.0031 L/D_H)   (10)

q″_co,4  = 0.098 (ρ_g/ρ_f)^{0.133} (σρ_f/(G²L))^{0.433} (L/D_H)^{0.27} · G h_fg / (1 + 0.0031 L/D_H)   (11)

q″_co,5  = 0.0384 (ρ_g/ρ_f)^{0.6} (σρ_f/(G²L))^{0.173} · G h_fg / (1 + 0.28(σρ_f/(G²L))^{0.233} L/D_H)  (12)

q″_co,13 = 0.234 (ρ_g/ρ_f)^{0.513} (σρ_f/(G²L))^{0.433} (L/D_H)^{0.27} · G h_fg / (1 + 0.0031 L/D_H)   (13)

Subcooling factors:

K6 = 1.043 / (4 C_Kc (σρ_f/(G²L))^{0.043})                                 (14)  [S: C_Kc]
K7 = (5/6) · [ 0.0124 + D_H/L ] / [ (ρ_g/ρ_f)^{0.133} (σρ_f/(G²L))^{0.333} ]   (15)
K9 = 1.12 · [ 1.52(σρ_f/(G²L))^{0.233} + D_H/L ] / [ (ρ_g/ρ_f)^{0.6}(σρ_f/(G²L))^{0.173} ]   (16)

Regime selection — the interesting part [V]:

q″_co = min(q″_co,2, q″_co,3, q″_co,4)     if ρ_g/ρ_f < 0.15
      = min(q″_co,2, q″_co,5, q″_co,13)    if ρ_g/ρ_f > 0.15               (17)

K     = max(K6, K7)                        if ρ_g/ρ_f < 0.15
      = max(K6, K7, K9)                    if ρ_g/ρ_f > 0.15               (18)

`ρ_g/ρ_f = 0.15` is a real, physically-motivated regime boundary — the density ratio at which the controlling CHF mechanism changes. It is a natural gate for a mixture-of- experts architecture, and it comes free from the physics rather than from cross-validation. Note also that the whole correlation is expressed in Bo, σρ_f/(G²L), ρ_g/ρ_f and L/D — exactly the four-group space of §2.2.

[S] flag: C_Kc is the Katto–Ohno length-dependent constant (a piecewise function of

L/D). Its value did not extract cleanly from the OCR and **must be read from Katto &

Ohno (1984), *Int. J. Heat Mass Transfer* 27(9):1641** before implementation.

### 5.6 Shah (1987)

The UCC (upstream conditions correlation) version [V]:

Bo = q″_CHF/(G h_fg) = 0.124 (D_H/L_E)^{0.89} (10⁴/Y)^n (1 − x_iE)         (19)

L_E  = L                        if x_in ≤ 0
     = L + D_H x_in/(4 Bo)      if x_in > 0                                (20)

x_iE = x_in                     if x_in ≤ 0
     = 0                        if x_in > 0                                (21)

Y = (G D_H c_p,f / k_f) · (ρ_f 2 g D_H / G²)  ...                          (22)  [S: partial OCR]

F3 = (1.25×10⁵/Y)^{0.833 x_eq}                                             (31)
c  = 0 if P/P_cr ≤ 0.6; 1 if P/P_cr > 0.6                                  (32)

Shah's own selection rule [V]: use the UCC method when Y ≤ 10⁶ or

L_E > 160/(P/P_cr)^{1.14}; otherwise use whichever version yields the lower Bo

(i.e. the conservative branch). Described in the source as "based on a vast amount of data

and can be applied to various fluids."

[S] flag: the full Y definition and Eqs. 23–30/33–36 did not extract cleanly.

### 5.7 Hall & Mudawar (2000) — the cleanest dimensionless correlation

This is the most important correlation in this document for our purposes, because it

is fully dimensionless, has only five constants, and — uniquely — its functional form was

*derived from observed parametric trends* rather than guessed and regressed. From Hall &

Mudawar, *IJHMT* 43:2605 [V]:

Outlet (local) conditions form:

Bo = C₁ We_D^{C₂} (ρ_f/ρ_g)^{C₃} [ 1 − C₄ (ρ_f/ρ_g)^{C₅} x_o ]            (7)

C₁ = 0.0332,  C₂ = −0.235,  C₃ = −0.681,  C₄ = 0.684,  C₅ = 0.832

with Bo = q″/(G h_fg), We_D = G²D/(ρ_f σ).

Inlet conditions form (obtained by substituting the energy balance, §1.5) [V]:

C₁ We_D^{C₂} (ρ_f/ρ_g)^{C₃} [ 1 − C₄(ρ_f/ρ_g)^{C₅} x_i* ]
Bo  =  ───────────────────────────────────────────────────────────────────    (11)
              1 + 4 C₁ C₄ We_D^{C₂} (ρ_f/ρ_g)^{C₃+C₅} (L/D)

All saturated properties at outlet pressure. Reported accuracy: MAE 10.3%, RMS 14.3%

for the inlet form — *more accurate than the CHF look-up table* on their database.

Validity [V]: 0.25×10⁻³ ≤ D ≤ 15×10⁻³ m, 2 ≤ L/D ≤ 200,

300 ≤ G ≤ 30,000 kg m⁻²s⁻¹, 1×10⁵ ≤ P ≤ 200×10⁵ N m⁻², −2.0 ≤ x_i ≤ 0.0,

−1.0 ≤ x_o ≤ 0.0 (subcooled outlet; ~85% of their subcooled database).

The authors' own claim about *why* it generalises is the thesis of this document [V]:

"Superiority of the correlations was attributed to the systematic development of the functional forms of the correlations from the CHF parametric effects; thus, re-optimization of the constants, when additional subcooled CHF data become available, is not expected to produce an appreciable increase in accuracy."

Read that again. A five-constant physics-derived form that does not improve when refitted on more data — that is the definition of a model that has captured the physics rather than the dataset. This is precisely the property our surface-wise and LOSO splits are trying to measure, and it is the target our proposed algorithm should aim at.

### 5.8 W-3 (Tong) and Bowring — [X] NOT TRANSCRIBED

Both are heavily cited (W-3 is the standard PWR DNB correlation; Bowring 1972 is a

standard tube correlation) and both appear in the ML-CHF literature as baselines. **No

clean primary source was obtained during this research** — the accessible sources

described them without giving full constants.

Do not write these into the paper from memory. Obtain:

Tong, L.S. (1967/1972), W-3 correlation — via the Westinghouse justification document

ML17254A842.pdf on nrc.gov, or Todreas & Kazimi *Nuclear Systems I*.

Bowring, R.W. (1972), AEEW-R789.

Tong's F-factor for non-uniform axial heat flux — MIT OCW 22.313J has a note on the

shape factor (chf_shape.pdf).

### 5.9 Analytical/mechanistic flow-boiling models

NUREG/KM-0011 §3.2 divides them into two families [V]:

(1) Annular film dryout models — mass balance on the liquid film; CHF = film

depletion. Originated by Hewitt and co-workers (Hewitt & Hall-Taylor 1970; Bennett et al.

1967). Models differ in their closures for **droplet entrainment, deposition, interfacial

friction and interfacial heat transfer**. Good for annular flow at medium-to-high pressure

and flow, void fraction > 50%.

(2) Bubbly layer models — CHF at low quality when the near-wall bubble layer becomes

so thick and vapour-saturated that liquid mixing with the cooler core stops. Tong (1965,

1968), Tong & Hewitt (1972), Weisman & Pei (1983), Ying & Weisman (1986). Good for

high-pressure, high-flow, low-quality/subcooled conditions.

NUREG's honest assessment, which we should quote [V]:

"CHF models tend to be less accurate than empirical correlations over the range of the correlation's database... the evaluation process is complex and time consuming."

There are 50+ such models; Weisman (1992) reviewed them.

(3) Liquid sublayer dryout — the third family, favoured in the modern DNB literature

because of direct experimental evidence in internally heated round tubes. DNB is caused by

complete evaporation of a thin superheated liquid layer beneath a vapour blanket sliding

along the wall. Heat balance (Zhao et al. 2019, Eq. 2) [V]:

CHF = ρ_f δ h_fg U_B / L_B

δ = liquid sublayer thickness, U_B = vapour blanket velocity, L_B = vapour blanket

length. The whole modelling problem reduces to closing these three quantities. The **Liu

model** is the most recent in the series, analysing instabilities at both the

sublayer/blanket and blanket/bulk interfaces. Reported limitations [V]: closure

relations exist essentially only for round tubes, and the model is **deficient at low

subcooling** — and it is outperformed by the purely data-driven LUT.

Note the pattern across §5.9: mechanistic models are more *transferable* in principle but less *accurate* in practice, because their closures are themselves empirical and tube-specific. This is the core tension our project sits in, and the honest framing for the paper's contribution.

• • •

## 6. Geometry-specific physics

### 6.1 Helical coils

Physically distinct from straight tubes because of **centrifugal force and secondary

flow. Hardik & Prabhu (2017), *Applied Thermal Engineering* 112:1223 [V]**:

Mechanism: phase separation, with low-density vapour driven to the inner side and

high-density liquid to the outer side of the curved tube; a thin liquid film forms

circumferentially under combined gravity and centrifugal forces. Result: large

circumferential wall-temperature variation and inner-side film dryout. Their IR

measurements confirm inner-half wall temperature exceeds outer-half.

Observed trends [V]: CHF increases with mass flux; decreases with quality; for the

same quality, decreases with increasing tube diameter *and* coil diameter. **In the

high-quality region CHF in a helical coil exceeds that in a vertical straight tube** — by

up to ~4× versus a straight *horizontal* tube.

Merilo-type dimensionless correlation (their Eq. 7) [V]:

q″/(G h_fg) = C · Re_l^{−0.34} · Z³ · Bo_d^{0.358} · (μ_l/μ_g)^{−2.18} · (L_h/d)^{−0.511}
              · (ρ_l/ρ_g − 1)^{1.27} · (1 − x_i)^{1.64}                    (7)

Re_l = G d/μ_l ,   Z = μ_l/(σ d ρ_l)^{0.5} ,   Bo_d = (ρ_l − ρ_g) g d²/σ ,
x_i = c_p(T_i − T_sat)/h_fg

with the constant C carrying all the geometry/regime information [V]:

C = 575    Merilo — high-pressure horizontal tubes                          (8)
C = 25.79  Baburajan et al. — low-pressure horizontal tubes                 (9)
C = 110    Hardik & Prabhu — low-pressure helical coils                     (10)

LUT-normalised form — the most useful result for us [V]:

CHF_coil / CHF_LookUpTable = 1.637 x + 0.568                               (11)

90% of data within 25%; RMS deviation 17%; average deviation −1.6%.

Physical explanation given [V]: at low quality, gravity dominates and coil CHF is

*below* a vertical tube; at high quality, liquid-film redeposition by secondary flow

dominates and coil CHF *exceeds* the vertical tube — hence the positive slope in x.

Two lessons. (a) Eq. (11) is a one-line, physically-explained transfer function from straight-tube physics to coil physics — exactly the kind of structure a generalised model should contain explicitly rather than relearn. (b) The C-only variation across Eqs. (8)–(10) shows the *dimensionless shape* transfers across geometry and pressure while only the *scale* changes. That is the strongest single piece of evidence in this document for the §9 architecture.

Our helical_coil_r123 source (257 rows) is the LOSO fold that went from R² = −409 to

+0.24 with physics normalisation. Adding curvature ratio d/D_coil as a feature and

Eq. (11) as a prior is the obvious next step.

### 6.2 Mini/micro-channels

Kandlikar's framework: surface tension, evaporation momentum, viscous shear and inertia

govern the flow patterns and CHF. Two new groups [S] (structure from secondary

sources; verify against Kandlikar 2001/2004 before use):

K₁ = (q″/(G h_fg))² (ρ_f/ρ_g)      — evaporation momentum / inertia
K₂ = (q″/h_fg)² D/(ρ_g σ)          — evaporation momentum / surface tension
K₃ = ...                            — evaporation momentum / viscous

with K₂/K₁ = We, K₃/K₁ = Re, K₂/K₃ = Ca. A high K₁ means evaporation momentum

dominates and will alter interface motion.

Kandlikar's physical picture for CHF: a large bubble on the surface subject to

momentum force (expanding), opposed by surface tension and hydrostatic forces;

when the balance fails the bubble advances and covers the surface → CHF. This is the same

force balance as his pool-boiling model (§4.3).

### 6.3 Pin fins / structured pool-boiling surfaces

No closed-form correlation obtained. Our pinfin_chf_water_fc72 source (175 rows) carries

fin_shape, fin_array, fin_width_um, fin_height_um, fin_spacing_um, coverage,

porosity, mbl_lateral_um, mbl_total_um — these are wickability geometry (§4.4)

and should be modelled through a wicking/capillary framework, plus the finite-heater-size

correction of §3.3, rather than as generic tabular features. This is the LOSO fold still

sitting at R² = −0.04.

• • •

## 7. What ML has and hasn't done — the gap, sourced

For the outline's §"Existing ML-based CHF prediction" and §"Research gap".

The benchmark context. The OECD/NEA established a Task Force on AI/ML for Scientific

Computing in Nuclear Engineering (2022), whose Phase 1 CHF exercise (NEA/WKP(2023)1,

Le Corre, Delipei, Wu, Zhao) covers regression, classification, VVUQ, dimensionality

reduction and anomaly detection, built on a US NRC-provided CHF database. [V] The

24,579-point NRC database at the core of it is the same nrc_groeneveld_24579pt source

that is 86% of our merged dataset.

The gap, in the words of the primary sources:

Zhao et al. (2019), MIT/ORNL [V]:

Standalone ML tools "can be prone to undesired, unphysical solutions due to their

purely data-driven nature and 'black-box' feature."

Of prior CHF-ML publications: none distinguished DNB from dryout; **none employed

cross-validation** to assess architectures; none evaluated hyperparameter sensitivity or

discussed regularisation.

Their hybrid ("grey-box") framework, in two flavours [V]:

Series: ML estimates intermediate variables that are closures in a physics model

(e.g. δ, U_B, L_B in the sublayer-dryout model).

Parallel (their choice, = physics-informed ML-aided): a fixed-structure prior model

ŷ_p = f(x) produces a baseline; ML is trained on the residual ε = y − ŷ_p;

prediction is ŷ_h = ŷ_p + ε̂_m.

Their feature vector is at least six variables: P, G, x_e, D_e, D_h, L_h — note both

equivalent and heated diameter. Metric [V]:

rRMSE = sqrt( (1/n) Σ ( (y⁽ⁱ⁾ − ŷ_h⁽ⁱ⁾)/y⁽ⁱ⁾ )² )                          (1)

Prior models used: Groeneveld 2006 LUT, and the mechanistic Liu model. They also propose

window-type extrapolation mapping to decide where new (expensive) experiments are

actually needed.

For non-circular channels, they note **heated diameter D_h rather than hydraulic

diameter D_e** is recommended for the diameter correction, "as the former feature better

describes vapor formation and development in subcooled and low-quality flow." [V]

Two direct implications for our pipeline. (a) We use diameter_mm with no D_h/D_e distinction — for the annulus (378 rows) and plate (48 rows) families this is a known, sourced modelling error. (b) Our repo already implements the *parallel* hybrid (residual learning on Biasi/Zuber) and found it fails on Split C because "the formula's error changed shape completely" outside the training range. That negative result is real and worth publishing — and it argues for *multiplicative* rather than *additive* residuals (§9), which is what the unified pipeline's log(CHF/baseline) target already does successfully.

• • •

## 8. Physical constraints any admissible CHF model must satisfy

This is the operational payload of the document. Each item is a testable assertion

derived from the physics above, usable as (a) a validation check, (b) a training penalty,

or (c) an architectural constraint. Nothing here requires new data.

### 8.1 Hard constraints (violation = the model is wrong)

| # | Constraint | Source |
| --- | --- | --- |
| C1 | CHF > 0 strictly, everywhere in the domain. | definition |
| C2 | Energy balance: x_o = x_i* + 4·Bo·(L/D) must hold exactly. x, L/D, G, q″ are not independent. | §1.5 [V] |
| C3 | ∂CHF/∂x ≤ 0 — CHF decreases monotonically with quality, in every regime and geometry examined. | §5, §6.1 [V] |
| C4 | ∂CHF/∂G ≥ 0 in flow boiling (all helical-coil and tube sources agree). | §6.1 [V] |
| C5 | As G → 0, the flow-boiling solution must approach the pool-boiling solution (Zuber/Kutateladze), not diverge. Biasi's G^{1/6} denominators violate this. | §5.4 [V] |
| C6 | As P → P_crit: σ → 0, h_fg → 0, ρ_f − ρ_g → 0, therefore CHF → 0. No feature may diverge here. | §2.1 |
| C7 | Dimensional homogeneity: predictions must be invariant under a change of unit system. Only satisfiable if the model operates on dimensionless groups. | §2.2 |

Constraint C6 is exactly the failure mode already documented in

docs/CHF_Physics_Approaches_Explained.md: the ln(1 − P/P_crit) feature exploded above

16,000 kPa and drove Split-C predictions to negative thousands. The physics said in

advance that this feature was inadmissible.

### 8.2 Soft constraints (strong expectations; deviations need explanation)

| # | Constraint | Source |
| --- | --- | --- |
| S1 | Pool-boiling CHF vs. pressure is non-monotonic with a maximum near `P/P_c ≈ 0.35`. | §4.6 [V] |
| S2 | Dimensionless K for saturated pool boiling on a large horizontal surface lies in 0.119–0.157 (Zuber's bracket), extending to ~0.13–0.2 across the literature. A model predicting K = 0.5 or K = 0.02 is reporting a different phenomenon. | §3.2, §4.6 [V] |
| S3 | CHF decreases with decreasing wettability (increasing contact angle); at α = 107° it can be half the hydrodynamic prediction. | §4.1 [V] |
| S4 | CHF increases with roughness where wicking is active, then saturates (no further gain for R_a ≳ 0.35 µm on superhydrophilic surfaces). Not monotonic without bound. | §4.2 [V] |
| S5 | CHF decreases slightly for orientation 0°→90°, then sharply 90°→180°. Three distinct mechanism regions. | §4.3 [V] |
| S6 | Diameter effect reverses sign at low mass flux (n < 0 for G < 250) and at high quality. A single global D^{−0.5} is wrong. | §5.3 [V] |
| S7 | Helical-coil CHF is below straight-tube at low quality and above it at high quality; the crossover is real, not noise. | §6.1 [V] |
| S8 | For finite heaters, Zuber applies only for width > 3λ_d (or > 12 capillary lengths, or > 2, depending on source). Below that, expect systematic deviation. | §3.3 [V] |
| S9 | Regime boundary at ρ_g/ρ_f = 0.15 (Katto–Ohno) separates two different controlling mechanisms. | §5.5 [V] |
| S10 | Wall thermal activity S matters in pool boiling (asymptote at S ≈ 10–85) and not in flow boiling. | §4.5, §1.4 [V] |

### 8.3 Data-integrity constraints specific to our dataset

| # | Issue | Evidence |
| --- | --- | --- |
| D1 | CHF label definition varies by source (4+ detection criteria). No detection_criterion column exists. | §1.3 |
| D2 | Contact angle: no column. Roughness: 0.6% coverage, and no peak-spacing S_m — Kim et al. Eq. (21) needs both. | §4.2 |
| D3 | No D_h vs D_e distinction; known to matter for annulus/plate rows. | §7 |
| D4 | No wall thickness → cannot compute thermal activity S for pool-boiling rows. | §4.5 |
| D5 | No flow-instability / throttling flag, though throttling causes "significant" CHF reduction and dissolved gas up to 30%. | §1.4 |
| D6 | Inter-laboratory reproducibility on nominally identical test sections is ~10% (European 1970 exercise, 8–10 labs; plus the U.S.S.R. 1984/85 study). This is the irreducible noise floor. Any reported MAPE below ~10% on pooled multi-source data is measuring overfitting, not accuracy. | NUREG §2.4 [V] |

D6 deserves emphasis in the paper. Our strategy-1 random-split results (R² ≈ 0.96–0.97) sit well inside the inter-laboratory reproducibility band. That is not a triumph; it is a sign the model is fitting source-specific bias. The surface-wise and LOSO numbers, unflattering as they are, are the physically meaningful ones.

• • •

## 9. Proposed algorithm — a physics-first architecture (discussion draft)

This section is a proposal for discussion, not an implemented design. It follows from

§8 and from the one empirical fact our own pipeline has already established: dividing by a

physics baseline before log-space training moved strategy 3 from R² = −16.4 to +0.22 and

LOSO helical_coil_r123 from −409 to +0.24. **The physics normalisation did more than

every model-selection decision combined.** The proposal is to take that principle

further rather than to add more model capacity.

### 9.1 The core idea

Stop predicting CHF. Predict a dimensionless correction to a physics scale:

CHF = Φ_physics(x) · exp( g_θ(π(x)) )

Φ_physics — a closed-form, regime-dispatched physical scale, computed from CoolProp

properties with zero fitted parameters.

π(x) — the dimensionless feature vector (§2.2), never raw engineering units.

g_θ — a learned, *bounded*, smooth correction in log space.

This is multiplicative, not additive — which matters, because the repo's additive

residual experiment (Approach 2) failed on Split C for exactly the reason the physics

predicts: an additive correction learned inside the domain has the wrong *magnitude* when

the baseline itself changes by an order of magnitude outside it. A multiplicative

correction is scale-free, which is why the log(CHF/baseline) target already works.

### 9.2 Layer 1 — the physics scale Φ_physics

Regime-dispatched, with smooth blending (Yagov's cube-root blend, §3.5, rather than

hard switches):

| Regime | Scale | Source |
| --- | --- | --- |
| Pool boiling, large heater | Zuber Eq. (15), K = 0.131 | §3.2 |
| Pool boiling, finite heater | × Lienhard–Dhir / size correction | §3.3 |
| Pool boiling, structured surface | × Kandlikar K(α, θ) and/or Kim K(α, R_a, S_m) | §4.3, §4.2 |
| Flow boiling, subcooled | Hall & Mudawar Eq. (7)/(11) | §5.7 |
| Flow boiling, saturated | Katto–Ohno Eq. (17) with the ρ_g/ρ_f = 0.15 gate | §5.5 |
| Helical coil | straight-tube scale × (1.637x + 0.568) | §6.1 |
| Any tube, D ≠ 8mm | × K1 = (8/D)^n, n from the Tanase regime table | §5.3 |

Every one of these is closed-form and needs no training data for the target fluid. That

is the property that makes cross-fluid and cross-surface generalisation possible at all.

### 9.3 Layer 2 — the dimensionless feature map π(x)

Replace (P, G, x, D, L) with:

π = [ Bo_ref,  We_D,  ρ_f/ρ_g,  P/P_c,  x,  L/D,  D/D_ref,
      Ja = ρ_f c_p ΔT_sub/(ρ_g h_fg),
      Bo_d = (ρ_f−ρ_g)g D²/σ,
      d/D_coil            (curvature, coils)
      cos α, θ, R_a/S_m, wicking flux   (surface, where available)
      S = H√((ρc_p k)_w)  (pool boiling, where available) ]

Rationale: NUREG's own general form (§2.2) says a tube CHF correlation *is* a function of

(ρ_f/ρ_g, D^{0.5}/(σρ_f)^{0.5}, x, D/D_ref). Katto–Ohno, Hall–Mudawar and Merilo are

all written in exactly this space. A model in this space can map water at 15 MPa and R123

at 1 MPa onto the same point; a model in raw units cannot.

### 9.4 Layer 3 — the constrained learner g_θ

Bounded output. g_θ ∈ [−ln 3, +ln 3] (a hard tanh), i.e. the correction may not

move the answer by more than ~3×. This replaces the current post-hoc

CHF_CLIP_MAX = 30,000 clip — which is a symptom-level fix for the ANN blow-up that

turned one fold's R² into −1,600,000 — with a structural guarantee.

Monotonicity in `x` (C3) and in G (C4), enforced either by a monotone architecture

(monotone lattice / constrained GBM) or by the autograd penalty already implemented in

the repo's collocation network.

Energy-balance consistency (C2) as a training penalty on x_o − x_i* − 4Bo(L/D).

Trust decay. g_θ → 0 as the input moves away from the training manifold, so

predictions gracefully fall back to pure physics out-of-domain instead of extrapolating

a learned correction. This directly addresses the documented Approach-2 failure.

### 9.5 Layer 4 — honest evaluation

Keep the existing four strategies, and add the §8 checks as a **physics-consistency

scorecard** reported alongside R²/RMSE/MAE/MAPE:

Fraction of predictions violating C1–C7.

Does the model reproduce the P/P_c ≈ 0.35 pool-boiling maximum? (S1)

Is predicted K inside 0.119–0.157 for saturated pool boiling on large flat heaters? (S2)

Does the helical-coil crossover (S7) appear?

Sign of the diameter effect at low G (S6).

A model that scores R² = 0.95 while violating C3 and missing S1 is worse than a model at R² = 0.85 that satisfies both, because only the latter can be trusted at a condition nobody has tested. That argument — not a leaderboard number — is the paper's contribution, and it maps directly onto the outline's Gap 4.

### 9.6 Honest assessment of this proposal

It will almost certainly lower strategy-1 (random split) scores. Constraints cost

in-distribution accuracy. That trade is the point, but it must be stated, not hidden.

It cannot manufacture the missing data (D1–D5). The surface-characteristics claims in

the outline remain unsupportable until contact angle and roughness coverage improve well

beyond 0.6%.

Several constants are still [S] or [X] (Katto–Ohno C_Kc, Shah's Y, Kandlikar's

K₁/K₂, W-3, Bowring, the Rahman wickability coefficient). These must be read from

primary sources before any of it is implemented or published.

The biggest risk is that Φ_physics is *itself* wrong for the exotic regimes (pin-fin,

R123 coils) — in which case we have moved the error rather than removed it. The

physics-consistency scorecard is what will detect that.

• • •

## 10. Sources

### 10.1 Read in full during this research (primary)

| Source | Location | What it provided |
| --- | --- | --- |
| Liang & Mudawar (2018), *IJHMT* 117:1352, "Pool boiling CHF Part 1: mechanisms, models, correlations" | fetched from Purdue | §2.1, §3 (all five mechanisms), §4 — the single richest source here |
| NUREG/KM-0011 (ML19029B306.pdf) | docs/references/ | §1.1–1.4, §1.6, §2.2, §5.1, §5.2, §5.9, D6 |
| Hall & Mudawar (2000), *IJHMT* 43:2605, Part II subcooled correlations | fetched from Purdue | §1.5, §5.7 |
| Hall & Mudawar (2000), *IJHMT* 43:2573, Part I database (Critical_heat_Flux.pdf) | docs/references/ | database scope, CHF definition |
| Hardik & Prabhu (2017), *Appl. Therm. Eng.* 112:1223 | docs/references/ | §6.1 |
| Tanase et al. (2009), *NED* 239:289, diameter effect | docs/references/ | §5.3 diameter exponent table |
| Zhao, Shirvan, Salko & Guo (2019), arXiv:1906.11124 | fetched | §7, §5.9(3) |
| NASA NTRS 20230009827, cryogenic CHF correlations | fetched | §5.5, §5.6 |
| arXiv:2203.15048, VVER rod bundle CHF methodology | fetched | §5.3 K1–K8 |
| NEA/WKP(2023)1 Phase 1 CHF benchmark specs | docs/references/ | §7 benchmark context |
| Rashidi et al. (2022), Applications_ML.pdf | docs/references/ | ML-boiling review, dimensionless input sets |
| scripts/chf_physics.py (this repo) | — | §5.4 Biasi as implemented, Zuber implementation |

### 10.2 Consulted but not fully mined

CHF_Look_Up_Table.pdf, ML22264A009.pdf (457 pp), Critical Heat Flux of Flowing Water.pdf,

Critical heat flux prediction through machine learning model for narrow.pdf,

Prediction of "critical heat flux" for supercritical water and CO2.pdf,

A Three-Stage Bayesian Transfer Learning Framework.pdf,

Boiling_pressure_drop...straight_tubes.pdf, CRITICAL_HEAT_FLUX_CHF_AND_POST-CHF_HEAT.pdf,

sample_reference.pdf, external_coil_tube_chf_appendix.pdf,

helical_coil_chf_research_data.pdf.

Extracted text for all of the above is in the session scratchpad; re-extract with

pypdf if needed.

### 10.3 Must be obtained before publication

Katto & Ohno (1984), *IJHMT* 27(9):1641 — the C_Kc constant. [S]

Shah (1987) — full Y definition and Eqs. 23–36. [S]

Kandlikar (2001), *J. Heat Transfer* 123(6):1071 — pool-boiling force balance

(the K form in §4.3 is verified; the derivation is not). [V]/[S]

Kandlikar (2004), *J. Heat Transfer* 126(1):8 — K₁, K₂, K₃ definitions. [S]

Tong W-3 and Bowring (1972) AEEW-R789 — full forms. [X]

Tong F-factor for non-uniform axial flux (MIT OCW 22.313J chf_shape.pdf). [X]

Rahman, Ölçeroğlu & McCarthy, *Langmuir* 2014 / *Sci. Rep.* 2017 — the linear

CHF–wickability coefficient. [S]

Groeneveld et al. (2007), *NED* 237:1909 — the K-factor formulas as given by the

original authors (§5.3 is transcribed from a secondary source that cites IAEA-TECDOC-1203).

Weisman & Pei (1983), *IJHMT* 26(10):1463 — bubble-crowding model equations. [X]

Liu et al. liquid sublayer dryout closures for δ, U_B, L_B. [X]

### 10.4 Web sources used

Liang & Mudawar pool boiling CHF review — https://engineering.purdue.edu/mudawar/files/articles-all/2018/2018-01.pdf

Hall & Mudawar subcooled CHF correlations — https://engineering.purdue.edu/mudawar/files/articles-all/2000/2000_2.pdf

NASA cryogenic CHF — https://ntrs.nasa.gov/api/citations/20230009827/downloads/CHF%20Paper_submitted.pdf

VVER rod bundle CHF methodology — https://arxiv.org/pdf/2203.15048

Zhao et al. physics-informed ML CHF — https://arxiv.org/pdf/1906.11124

Kandlikar pool boiling CHF model (abstract) — https://asmedigitalcollection.asme.org/heattransfer/article-abstract/123/6/1071/445633/A-Theoretical-Model-to-Predict-Pool-Boiling-CHF

Rahman et al. wickability (abstract) — https://pubs.acs.org/doi/full/10.1021/la5030923

CHF wickability paradigm (abstract) — https://www.nature.com/articles/s41598-017-05036-2

Katto & Ohno improved generalized correlation (abstract) — https://www.sciencedirect.com/science/article/abs/pii/001793108490276X